### March 17, 2026

# To:	All Interested Bidders

## Re:	Bid Solicitation # 25DPP01178

T2854 – License Plate Materials, Manufacturing, Printing and Related Services for the New Jersey Motor Vehicle Commission (NJMVC)

Revised Question and Answer Due Date*: April 8, 2026 (2:00 p.m. Eastern Time)

# Quote Submission Due Date: December 19, 2025 (2:00 p.m. Eastern Time)

# REVISED Quote Submission Due Date:  January 30, 2026 (2:00 p.m. Eastern Time)

# REVISED Quote Submission Due Date: April 24, 2026 (2:00 p.m. Eastern Time)

# Bid Amendment #4

The following constitutes Bid Amendment #4 to the above referenced Bid Solicitation:

This Bid Amendment includes:

- Answers to questions from the Electronic Question and Answer Period that concluded on November 19, 2025;

- Revisions to the Bid Solicitation (from Part 1 and Part 2 of the Bid Amendment);

Please note that for all additions, deletions, clarifications and modifications to the Bid Solicitation, please refer to the Revised Bid Solicitation titled, “T2854 Revised Bid Solicitation 3.17.2026”

- Revised State-Supplied Price Sheet titled “T2854 Revised State-Supplied Price Sheet 3.17.2026”.

- *The Question and Answer Period has been re-opened to address any follow up questions resulting from the questions and answers in this Bid Amendment.  Please thoroughly review the questions and answers herein.  Do not submit any questions that were previously asked and answered through this Bid Amendment.  Any such questions will not be addressed.

It is the sole responsibility of the Bidder to be knowledgeable of all of the additions, deletions, clarifications, and modifications to the Bid Solicitation and/or the New Jersey Standard Terms and Conditions relative to this Bid Solicitation as set forth in all Bid Amendments.

All other instructions, terms, and conditions of the Bid Solicitation shall remain the same.

## Bid Solicitation # 25DPP01178

T2854 – License Plate Materials, Manufacturing, Printing and Related Services for the New Jersey Motor Vehicle Commission (NJMVC)

#### Part 1 - Answers to Questions

Where applicable, each question references the appropriate Bid Solicitation section.

| # | Bid Solicitation Section Reference | Question (Bolded) and Answer |
| --- | --- | --- |
| 1 | PDF Documents | Question: When trying to open a Portable Document Format (PDF), I am receiving an error message and am unable to access the PDF. How should I proceed? Answer: It is recommended that users utilize Microsoft Edge to view PDFs. Certain PDF forms cannot be opened in the Chrome or Firefox built in PDF viewers that are enabled by default. If using Chrome or Firefox it is recommended that the file is downloaded instead by clicking the download button in the upper right-hand corner of the browser after an error message is received. An alternative option is to disable the built in PDF viewers in Chrome or Firefox. |
| 2 | Quote Submission | Question: Should I check that I have properly completed and attached all of the required documents with my submitted Quote? Answer: Yes. Prior to the Quote Opening Deadline, a Bidder should review the documents comprising the submitted Quote to ensure that all of the required documents have been completed and attached. Oftentimes a Quote is deemed non-responsive because a Bidder fails to attach a required form or attaches a blank form. To review a submitted Quote, a Bidder should log into NJSTART and review each document attached to the Quote submission. Instructions on how to review a submitted Quote can be found in the Quick Reference Guide entitled How to Review a Submitted Quote, which is available on the NJSTART Vendor Support Page under Vendor Reference Guides. |
| 3 | Quote Submission | Question: Please confirm a Bidder may deliver a hardcopy quote instead of submitting a response electronically. Answer: Please be advised that all Quotes shall be submitted electronically through NJSTART. |
| 4 | Exceptions | Question: Will the State allow Bidders to take exception to any of the Standard Terms and Conditions? Answer: As stated in Bid Solicitation Section 2.2, Exceptions to the State of NJ Standard Terms and Conditions (SSTC): “Questions regarding the State of New Jersey Standard Terms and Conditions and exceptions to mandatory requirements MUST be posed during the Electronic Question and Answer period and shall contain the Bidder’s suggested changes and the reason(s) for the suggested change(s).” Moreover, as stated in Bid Solicitation Section 3.11, Bidder proposed terms or conditions that conflict with those contained in the State of New Jersey Standard Terms and Conditions may render a Quote non-responsive. |
| 5 | Specification Conformance Certification, Bid Solicitation Section 3.21 | Question: Is a Bidder required to provide the signed Specification Conformance Certification with the Quote? Answer: Yes, Bidders are required to provide, with its Quote, the signed Specification Conformance Certification as indicated in Bid Solicitation Section 3.21. Bid Solicitation Section 3.21 states: “The Bidder shall submit with its Quote the Specification Conformance Form. By submitting this form, the Bidders certifies that all items offered in response to this Bid Solicitation conform to the requirements set forth in the Bid Solicitation.” |
| 6 | State-Supplied Price Sheet Instructions, Bid Solicitation Section 3.36 | Question: Do I have to complete all of the required fields on the State-Supplied Price Sheet for each price line bid? Answer: Yes. |
| 7 | Group Awards | Question: Do I have to bid on all price lines within the group to be considered for an award? Answer: Yes, as stated in Bid Solicitation Section 1.1, "Bidders must bid on all five (5) Price Lines (1 through 5) on the State-Supplied Price Sheet in order for its Quote to be considered responsive to the requirements set forth in Bid Solicitation Section 8.9.2." If a Bidder fails to bid on all five (5) Price Lines (1 through 5) or is determined to be non-responsive for any of the five (5) Price Lines, their Quote shall be considered non-responsive. |
| 8 | ## Price Adjustments | Question: Will quarterly price adjustments be considered once the Contract awards are made? Answer: Yes, please refer to Bid Solicitation Sections 4.9 and 4.15. |
| 9 | Background, Bid Solicitation Section 1.2 Estimated Ordering Quantity, Bid Solicitation Section 4.17 | Question: Section 1.2 - states NJMVC processes approximately 1,500,000 license plate pairs, but in Section 4.16 it states NJMVC administers approximately 1,100,000 license plate pairs. Can you please confirm the volume of plates per year? Answer: NJMVC administers approximately 1,500,000 license plate pairs annually. Bid Solicitation Section 4.17 has been updated to change the quantity from 1,100,000 to 1,500,000. Please refer to Part 2 of this Bid Amendment and the revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026”. |
| 10 | State-Supplied Price Sheet Instructions, Bid Solicitation Section 3.36 | Question: Could the State please confirm that this statement, “each bidder is required to hold its pricing firm through the issuance of the contract” is intended to indicate that a bidder must hold their pricing firm between the time of bid submission and contract award/issuance date. Answer: Yes, your interpretation of the language is correct. Each Bidder is required to hold its pricing firm through issuance of the Contract, meaning that the Contractor must hold its prices firm through the effective date (begin date) of the Contract. |
| 11 | State-Supplied Price Sheet Instructions, Bid Solicitation Section 3.36 ## Reserved, ## Bid Solicitation Section 4.9 ## Price Adjustments for Price Lines 1 through 5, ## Bid Solicitation Section 4.16 | Question: While there is a provision to adjust aluminum pricing (up or down) on a quarterly basis, there does not appear to be a mechanism for price adjustments related to reflective sheeting, printing or other required materials during the term of the contract. Would the State please consider including an annual price review provision within this contract to allow for potential price adjustments, with appropriate supporting documentation? Answer: For all price adjustment requests, including but not limited to aluminum alloy, printed reflective sheeting, printing/labor costs and other required materials, the Contractor may request price adjustments which will be considered by the Division provided that the Contractor submits proper justification to support the price increase and specifically indicates to which price line(s) the requested price increase is applicable. The language in Bid Solicitation Section 4.16 has been updated. Bid Solicitation Section 4.9 has been removed in its entirety. Please refer to Part 2 of this Bid Amendment and the revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026”. |
| 12 | State-Supplied Price Sheet Instructions, Bid Solicitation Section 3.36 Finished Plate Dimensions, Bid Solicitation Section 4.1.1 Agency Net Use Line – Price Line 9 – For the Payment of Blanking Line Repair Services, Maintenance and/or Replacement Parts, Bid Solicitation Section 4.13 | Question: This section states that the printed reflective sheeting and aluminum alloy coil stock are bonded and paired plate sets assembled and finished in an established production process at the DEPTCOR Bayside Correctional Facility – Tag Shop, Leesburg, New Jersey. - Please confirm that DEPTCOR owns the entire blanking line and that the Contractor is not required to supply any equipment to DEPTCOR for this project. Answer: DEPTCOR owns the blanking machine, Blanker NO#--Federal – Model-60- OBI & S/N – 60-633 & P/N-P-6262. Bid Solicitation Section 4.1.1 states, in pertinent part: “*DEPTCOR utilizes a Federal brand machine- blanking press Model Number 60-OBI, manufactured by the Federal Press Company.” Question: - Given that the Contractor must warranty the finished license plate product, please list the components of the blanking line (blanking press, uncoiler, straightener, applicator, electronic roll feed, blanking dies, etc.) and the make and model numbers of each. Answer: The Contractor must provide the equipment listed in the chart in part “c” of this answer. - Is the successful bidder responsible for blanking line maintenance? If yes, can the State provide a line item on the price sheet for blanking line maintenance? Answer: Yes, the Contractor is responsible for annual maintenance. An Agency Net Use Line (Price Line 9) will be added to the State-Supplied Price Sheet for blanking line/machine repair services, maintenance and/or replacement parts. Bid Solicitation Section 4.13 has been added to reflect this modification. Please refer to Part 2 of this Bid Amendment, the revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026”, and the revised State-Supplied Price Sheet titled “T2854 Revised State-Supplied Price Sheet 3.17.2026.” Please see the chart below detailing: the components of the blanking line/machine, the make/model number of each component, and the party responsible for the maintenance of same. Blanking Line/Machine Component; Make and Model Number; Party Responsible for Maintenance. Press; Federal 60 Ton Serial #60-633; Contractor. Roll Feed; CWP Asset #2046292; Contractor. Steering Table; Serial #750 Asset #2009950; Contractor. Electronic Roll Feed Panel; Serial #650 Asset #2009949; Contractor. Applicator; Serial #542 Asset #2009948; Contractor. Straightener; See Unwind as it is combined with the Unwind; N/A. Unwind; CWP - 12CRS Combo Serial #C27717 Asset #2046293; Contractor. Conveyor 1; Dorner 6200 8 inch Serial #611538-1-1-1 Asset #2046291; Contractor. Slug Collector; Cincinnati Dustmaster 200S; Contractor. Die 1; Branch Mfg. Serial #TN1691 No Asset tag; Contractor. Die 2; Branch Mfg. Serial #TN1464 Asset #2046290 (MC Die and spare Die steels); Contractor. |
| 13 | Aluminum Alloy Coil Stock (Price Line 1), Bid Solicitation Section 4.1 | Question: This section lists a number of different tempers (hardness) that are available to be provided by the contractor. A standard aluminum temper in the industry is 3105 H-21. Can NJMVC confirm that this temper will also be acceptable? Answer: Aluminum temper 3105 H-21 is acceptable. The chart in Bid Solicitation Section 4.1 has been updated to reflect this addition. Please refer to Part 2 of this Bid Amendment and the revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026”. |
| 14 | Aluminum Alloy Coil Stock (Price Line 1), Bid Solicitation Section 4.1 Finished Plate Dimensions, Bid Solicitation Section 4.1.1 | Question: This section states the width of aluminum coil is to be 12.125 inches. In section 4.1.1, it states the dimensions of the finished plate should be 12 inches wide. Can the State verify the exact width (12 inches exact, 12.125 inches exact, or other) of aluminum coil required by DEPTCOR? Note: This question has been paraphrased in the interest of readability and clarity. Answer: The aluminum alloy coil must be 12 inches exact. The chart in Bid Solicitation Section 4.1 has been updated to reflect this correction. Please refer to Part 2 of this Bid Amendment and the revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026”. |
| 15 | Aluminum Alloy Coil Stock (Price Line 1), Bid Solicitation Section 4.1 | Question: This section under “Material Preparation” states that the conversion coating on aluminum shall be chrome free. Can the State confirm that trivalent chrome conversion coating (industry standard) is acceptable? Answer: Yes, trivalent chrome conversion coating is acceptable. Bid Solicitation Section 4.1 has been updated to reflect this addition. Please refer to Part 2 of this Bid Amendment and the revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026”. |
| 16 | State-Supplied Price Sheet Instructions, Bid Solicitation Section 3.36 | Question: Please confirm whether shipping charges for the transportation of aluminum can be billed separately, at actual cost, during the term of the contract. Shipping costs are subject to fluctuation due to factors such as fuel surcharges and general freight cost increases. Allowing billing at actual cost would provide the best value to the State and help mitigate the need for bidders to include unnecessary price premiums in their bid pricing to account for potential future freight cost increases. Answer: The Bidder must include additional costs, including but not limited to fees for shipping, in its pricing submitted with its Quote. Bid Solicitation Section 3.36, Step 3 states, in pertinent part (emphasis added in bold): “Step 3 – For Price Lines 1, 2, 3, 4 and 5: The Bidder shall insert a firm, fixed price in US dollars in the field titled “Unit Price” on the State-Supplied Price Sheet. If a Bidder leaves the “Unit Price” field blank for Price Lines 1, 2, 3, 4 and 5, its Quote shall be deemed non-responsive. For Price Line 1: Bidders shall insert a firm, fixed price in US Dollars per metric ton in the "Unit Price" field. For Price Lines 2 and 3: Bidders shall insert a firm, fixed price in US Dollars per Plate Pair in the "Unit Price" field. For Price Lines 4 and 5: Bidders shall insert a firm, fixed price in US Dollars per Single Plate in the "Unit Price" field. Any additional costs, including freight, must be included in the pricing offered on the State-Supplied Price Sheet.” |
| 17 | Aluminum Alloy Coil Stock (Price Line 1), Bid Solicitation Section 4.1 | Question: Please confirm the reason for specifying that license plates are to be embossed to a maximum of 0.090 inches. Question: Is this an outdated specification? Question: It is understood that all the license plates are now digitally printed flat and no longer require male and female dies for embossing the alphanumeric (other than an embossed rim flange). Could the State please clarify this requirement? Answer: This Bid Solicitation has been revised to reflect that NJMVC will only accept flat digitally printed license plates. As such, references to the embossed technique/embossed license plates have been removed from the Bid Solicitation. Please refer to Part 2 of this Bid Amendment and the revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026”. |
| 18 | Aluminum Alloy Coil Stock (Price Line 1), Bid Solicitation Section 4.1 Purpose, and Intent and Background, Bid Solicitation 1.1 Estimated Ordering Quantity, Bid Solicitation 4.17 | Question: Please indicate the volume of aluminum required for this bid, for both 12 inch by 7 inch plate sizes per year. Answer: Please see below for an average of the volume of aluminum ordered for the past three years: 2023 491,646 pounds = 246 coils; 2024 674,195 pounds = 337 coils; and 2025 731,415 pounds = 365 coils. However, the above averages, as well as any other mention of estimated quantities/volumes in the Bid Solicitation, are estimates only and shall not be relied upon as any guarantee as to the minimum, average or maximum volume of purchases to be made under this Contract. Bid Solicitation Sections 1.1 and 4.17 state, in pertinent part: “The State makes no representation and provides no guarantee as to the minimum, average or maximum volume of purchases to be made under this Contract.” |
| 19 | Aluminum Alloy Coil Stock Delivery Preparation, Bid Solicitation Section 4.1.2 | Question: Please indicate the approximate scrap metal rate for aluminum and reflective sheeting. Answer: The approximate scrap metal rate for aluminum and reflective sheeting is less than 1.5%. |
| 20 | Finished Plate Dimensions, Bid Solicitation Section 4.1.1 | Question: Please confirm that the motorcycle size plates are exactly 4 inches by 7 inches. Answer: Yes, the size of the motorcycle size license plates are 7 inches wide by 4 inches long. Bid Solicitation Section 4.1.1 states, in pertinent part: “Motorcycle Plate – Single Plate (Price Line 5): Dimensions: 7 inches wide X 4 inches long” |
| 21 | Finished Plate Dimensions, Bid Solicitation Section 4.1.1 | Question: Please confirm the coil size of aluminum for motorcycle license plates is exactly seven (7) inches wide. Answer: Yes, the coil size of aluminum for motorcycle license plates is 7 inches wide. Please refer to Bid Solicitation Section 4.1.1. |
| 22 | Graphic Design Work, Bid Solicitation Section 4.2.1 | Question: Please confirm that NJMVC or DEPTCOR own all the graphics for all images that are to be printed by the Contractor. Answer: Yes, all images and graphics for license plates are the property of NJMVC. Bid Solicitation Section 4.2.1 states, in pertinent part: “All images and graphics for license plates are the property of the NJMVC and shall remain the property of the NJMVC following Contract expiration or Contract termination for cause or convenience.” |
| 23 | Graphic Design Work, Bid Solicitation Section 4.2.1 | Question: Please confirm that NJMVC or DEPTCOR will provide all of the graphics and artwork files required to be printed by the Contractor. Answer: Yes, NJMVC will provide to the awarded Contractor all graphics and artwork files to be printed by the Contractor. Bid Solicitation Section 4.2.1 has been updated to reflect this addition. Please refer to Part 2 of this Bid Amendment and the revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026”. |
| 24 | Graphic Design Work, Bid Solicitation Section 4.2.1 | Question: Please confirm that NJMVC or DEPTCOR will provide all sequence printing rules and configurations for each plate type to be printed. Answer: Yes, NJMVC will provide the initial license plate sequencing and increment rules. Any subsequent changes to the initial increment rules will be provided only if a series runs out or changes. Bid Solicitation Section 4.2.1 states, in pertinent part: “For all standard issue license plates that run in a continuous alpha-numeric series, the NJMVC will prepare the series order for batch processing as follows, example: AAA-001 - BBB-999. Unless otherwise excluded by law, and in conjunction with NJMVC policy, the Contractor shall review the plate series for potentially offensive, profane and or derogatory language, acronyms, words** for deletion/destruction. The Contractor must create and maintain and electronic listing of these plates after the initial series is handed over by NJMVC.” |
| 25 | Graphic Design Work, Bid Solicitation Section 4.2.1 | Question: Would the State please confirm its ownership of the digitally printed font that is used on its license plate sequences? Answer: Please refer to the answer to question #22 above. |
| 26 | Graphic Design Work, Bid Solicitation Section 4.2.1 | Question: Please confirm that the NJMVC is ultimately responsible for making all determinations as to what sequences are deemed potentially offensive, profane and/or derogatory language, acronyms, etc. Answer: Yes, a list of prohibited combinations (“omit” list) will be provided by NJMVC to the Contractor at the beginning of the project, as well as any subsequent changes to that list. Should the Contractor question any language and/or combinations, which are not included on the prohibited combinations list (“omit” list) provided by NJMVC, the Contractor must contact NJMVC for a final determination. Bid Solicitation 4.2.1 states, in pertinent part: “Unless otherwise excluded by law, and in conjunction with NJMVC policy, the Contractor shall review the plate series for potentially offensive, profane and or derogatory language, acronyms, words** for deletion/destruction. The Contractor must create and maintain and electronic listing of these plates after the initial series is handed over by NJMVC. **For plates that contain potentially offensive, profane and/or derogatory language, acronyms or words, the NJMVC will provide an “omit” list to the Contractor. The Contractor must review submitted plate orders for any offensive language/acronyms to avoid printing. If printed in error, the Contractor must destroy and report the destruction to the NJMVC within five (5) Business Days.” |
| 27 | Specifications, Bid Solicitation Section 4.2.2 Quality Conformance, Bid Solicitation Section 4.3.1 Agency Net Use Line – Price Line 8 – For the Payment of Expedited Delivery Charges, Bid Solicitation Section 4.12 | Question: Please indicate how NJMVC/DEPTCOR handles remakes of printed sheeting that become unusable or damaged during the finishing process at DEPTCOR. Answer: DEPTCOR will provide a list to the NJMVC and the Contractor, detailing what license plates need to be re-made. Any unusable or damaged printing sheeting which occurred during the machinery and/or finishing processing as the result of DEPTCOR will not be charged to the Contractor. Question: What is the expected turnaround time required for remakes of printed sheeting? Answer: Once an error is discovered and conveyed to the Contractor, the expected turnaround time for the remakes of printed sheeting will be no more than five (5) business days. Question: Are there rush orders? Please describe the desired process. Answer: There are occasions where personalized license plates are ordered on an expedited basis due to the discovery of errors. Expected turnaround time for the expedited orders will be no more than five (5) business days. An Agency Net Use Line (Price Line 8) will be added to the State-Supplied Price Sheet. Bid Solicitation Section 4.12 has been added to reflect this modification. Please refer to Part 2 of this Bid Amendment, the revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026”, and the revised State-Supplied Price Sheet titled “T2854 Revised State-Supplied Price Sheet 3.17.2026.” |
| 28 | Specifications, Bid Solicitation Section 4.2.2 | Question: Please confirm that any remakes of printed sheeting, due to DEPTCOR processing/application issues, i.e. that are not the fault of the Contractor, can be invoiced as another instance of printing, and shipping of remakes will be prepaid by the Contractor and invoiced at cost to NJMVC. Answer: If an error and/or damage occurs that is not the fault of the Contractor, the remake will be requested by NJMVC/DEPTCOR as stated in the answer to number 27 above and will be paid for by NJMVC/DEPTCOR utilizing the Price Line (Price Lines 1 through 5) to re-order the appropriate material. |
| 29 | Flexibility – Reflective Film Quality, Bid Solicitation Section 4.2.3 Current Manufacturing Process, Bid Solicitation Section 4.8 | Question: Please advise as to when/where in the DEPTCOR current manufacturing process the rim flange is embossed. Question: Is this done in-line at the blanking/cropping line, or is this done as a separate embossing process after blanking? Answer: Please refer to the answer to question #17 above. |
| 30 | Quality Conformance, Bid Solicitation Section 4.3.1 Printed Reflective Sheeting, Bid Solicitation Section 4.3.3.1 | Question: This section states the sheeting provided by the Contractor shall maintain a useful product of life for a five (5) year period. The performance of the reflective sheeting is highly dependent on the application of the sheeting to the aluminum substrate. Can the State please confirm that a failure of a license plate due to the application of the sheeting to aluminum will be replaced at the responsibility of DEPTCOR? Answer: Please see the answers to question #s 27 and 28 above. |
| 31 | Retroreflective Characteristics, Bid Solicitation Section 4.3.3.4 | Question: This section has a chart that illustrates “Minimum Values for Coefficient of Retroreflections of Reflective Sheeting per ASTM E-810”. Can NJMVC confirm if it was an oversight to put the units as degrees and if it should instead be cd/lux/m^2? Note: This question has been paraphrased in the interest of readability and clarity. Answer: Yes, this was an oversight. The unit of measure listed in the referenced chart should be listed in cd/lux/m^2. The chart in Bid Solicitation Section 4.3.3.4 has been updated to reflect this correction. Please refer to Part 2 of this Bid Amendment and the revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026”. |
| 32 | Disaster Recovery Plan “A” – Aluminum Ally Coil Stock and Printed Reflective Sheeting Supply, Bid Solicitation Section 4.4.1 | Question: This section makes notes of DEPTCOR’s Delaware Disaster Recovery site. Can the State please share the capability of this disaster recovery site and the address? Note: This question has been paraphrased in the interest of readability and clarity. Answer: All references to the Disaster Recovery site located in Delaware have been deleted from the Bid Solicitation. Please refer to Part 2 of this Bid Amendment and the revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026”. The purpose of the disaster recovery site is to ensure that in the event of any natural or man-made disaster, or the Contractor’s primary location is rendered unusable for any reason, all Contract deliverables and performance thereof continues without interruption. In the event of such disaster, determination or rendering of the Contractor’s primary business location as “unusable”, the Contractor may be required to implement its State Contract Manager-approved disaster recovery plan to continue uninterrupted operations and satisfy all terms and conditions of any Contract resulting from this Bid Solicitation within twenty-four (24) hours. As such, the Contractor would be responsible for full assembly of license plates including application of aluminum and reflective sheeting for any disaster recovery needed from DEPTCOR. Question: Is this an active disaster recovery site for New Jersey? Answer: All references to the Disaster Recovery site located in Delaware have been deleted from the Bid Solicitation. Please refer to Part 2 of this Bid Amendment and the revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026”. Question: a. Is it anticipated that this will remain a disaster recovery site for New Jersey? b. Will this function completely switch to the Contractor? Note: This question has been paraphrased in the interest of readability and clarity. Answer: a. No. and b. Yes. |
| 33 | Disaster Recovery Plan “A” – Aluminum Alloy Coil Stock and Printed Reflective Sheeting Supply, Bid Solicitation Section 4.4.1 Disaster Recovery Plan “B” – Complete Manufacture of Plates, Bid Solicitation Section 4.4.2 Mobilization Plan, Bid Solicitation Section 3.28 | Question: This section refers to the submission of the draft Disaster Recovery Plan B within 30 (thirty) days following initial contract kick-off. Please confirm that the State requires respondents to already have a fully owned and operational, non-correctional backup disaster recovery facility in place at time of bid submission, rather than establishing such a facility as part of the post award implementation plan. Answer: After Contract award, the Discovery Recover Plans are required. The Disaster Recovery Plan “A” is to identify locations, systems, procedures and processes to ensure, in the event of any natural or man-made disaster, or the Contractor’s primary location is rendered unusable for any reason, all Contract deliverables and performance thereof continues without interruption. The Disaster Recovery Plan “B” is a secondary plan to be utilized in the event a natural or man-made disaster renders or otherwise determines that DEPTCOR’s Tag Shop (Leesburg, New Jersey location) and DEPTCOR’s current disaster recovery site are inoperable. The Disaster Recovery Plan “B” must be submitted within thirty (30) calendar days following any initial project of contract kick off meeting and prior to its disaster recovery plan “B” implementation. Both plans must be approved by NJMVC prior to implementation. Please refer to Bid Solicitation Sections 4.4.1 and 4.4.2. However, a mobilization plan should be submitted with the Bidder’s Quote. The mobilization plan should detail the Bidder’s plan/detailed timetable to have a facility operational from the time of notification of award and lasting no longer than twelve (12) weeks. In other words, the Bidder should have all equipment and facilities ready to commence work no later than twelve (12) weeks from the date of Contract award notification. Bid Solicitation Section 3.28 states, in pertinent part: “It is essential that the State have quick use of the functionality this Contract is to provide. Therefore, each Bidder should include as part of its Quote, a mobilization plan, beginning with the date of notification of Contract award and lasting no longer than twelve (12) weeks. Such mobilization plan should include the following elements: - A detailed timetable for the mobilization period of twelve (12) weeks. This timetable should be designed to demonstrate how the Bidder will have the personnel and equipment it needs to begin work on the Contract up and operational from the date of notification of award.” |
| 34 | Mobilization Plan, Bid Solicitation Section 3.28 Delivery Schedule, Bid Solicitation Section 4.5 | Question: This section indicates that the Contractor’s first delivery shall be no later than forty-five (45) calendar days following the approval of the samples by the NJMVC, but Section 3.28 indicates that the Contractor has 12 weeks for the mobilization period. These two dates seem to contradict. Can the State please clarify? Answer: The mobilization plan is separate and apart from the delivery schedule. The mobilization plan should detail the Bidder’s plan/detailed timetable to have a facility operational from the time of notification of award and lasting no longer than twelve (12) weeks. In other words, the Bidder should have all equipment and facilities ready to commence work no later than twelve (12) weeks from the date of Contract award notification. Bid Solicitation Section 3.28 states, in pertinent part: “It is essential that the State have quick use of the functionality this Contract is to provide. Therefore, each Bidder should include as part of its Quote, a mobilization plan, beginning with the date of notification of Contract award and lasting no longer than twelve (12) weeks. Such mobilization plan should include the following elements: - A detailed timetable for the mobilization period of twelve (12) weeks. This timetable should be designed to demonstrate how the Bidder will have the personnel and equipment it needs to begin work on the Contract up and operational from the date of notification of award;” The delivery schedule requirement listed in Bid Solicitation Section 4.5 is the turnaround time for the initial order to be completed upon receipt of order once the facility is fully operational. Please refer to Bid Solicitation Section 4.5. |
| 35 | Delivery Schedule, Bid Solicitation Section 4.5 | Question: This section indicates that the Contractor shall be accountable for all printed reflective sheeting from the place of printing to the point of delivery. Please confirm that the Contractor is not responsible for arranging or shipping finished plates from DEPTCOR to NJMVC. Answer: The Contractor is responsible for the arrangement of shipping of the aluminum alloy coil stock and printed reflective sheeting directly to the DEPTCOR Tag Shop. The Contractor also manages the delivery of the finished project from DEPTCOR to the MVC warehouse located on North Olden Avenue, Trenton, New Jersey. Bid Solicitation Section 4.5.has been updated to reflect this modification. Please refer to Part 2 of this Bid Amendment and the revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026”. |
| 36 | Delivery Schedule, Bid Solicitation Section 4.5 | Question: Please provide an outline of all DEPTCOR security measures required for delivery of sheeting and aluminum to the DEPTCOR facility. Answer: All drivers are required to undergo a background check prior to delivery. MVC currently coordinates the background checks with the Contractor. The Contractor will be notified of the procedure for background checks after award. As a matter of reference, currently MVC utilizes IdentoGO to conduct the background checks and the current Contractor assumes all costs associated with same. The deliveries take approximately fifteen (15) minutes for unloading and loading. Weapons, cell phones, passengers and/or pets are not permitted. Trucks will be searched both driving into the facility and driving out of the facility. For additional information, the Contractor must contact DEPTCOR directly to arrange all aspects of delivery, including but not limited to security measures. Bid Solicitation Section 4.5 states, in pertinent part (emphasis added in bold.): “The Contractor must contact DEPTCOR to coordinate all aspects of the delivery, including, but not limited to, the delivery timeframes and security measures for delivery. Upon Contract award, the Award Summary for T2854 will list the specific DEPTCOR contacts and e-mail addresses.” Question: How much time is typically required to get printed sheeting and aluminum through security at DEPTCOR? Answer: All drivers are required to undergo a background check prior to delivery. MVC currently coordinates the background checks with the Contractor. The Contractor will be notified of the procedure for background checks after award. As a matter of reference, currently MVC utilizes IdentoGO to conduct the background checks and the current Contractor assumes all costs associated with same. The deliveries take approximately fifteen (15) minutes for unloading and loading. Weapons, cell phones, passengers and/or pets are not permitted. Trucks will be searched both driving into the facility and driving out of the facility. For additional information, the Contractor must contact DEPTCOR directly to arrange all aspects of delivery, including but not limited to security measures. Bid Solicitation Section 4.5 states, in pertinent part (emphasis added in bold.): “The Contractor must contact DEPTCOR to coordinate all aspects of the delivery, including, but not limited to, the delivery timeframes and security measures for delivery. Upon Contract award, the Award Summary for T2854 will list the specific DEPTCOR contacts and e-mail addresses.” Question: What requirements are placed upon shipping companies delivering product to DEPTCOR? Answer: All drivers are required to undergo a background check prior to delivery. MVC currently coordinates the background checks with the Contractor. The Contractor will be notified of the procedure for background checks after award. As a matter of reference, currently MVC utilizes IdentoGO to conduct the background checks and the current Contractor assumes all costs associated with same. The deliveries take approximately fifteen (15) minutes for unloading and loading. Weapons, cell phones, passengers and/or pets are not permitted. Trucks will be searched both driving into the facility and driving out of the facility. For additional information, the Contractor must contact DEPTCOR directly to arrange all aspects of delivery, including but not limited to security measures. Bid Solicitation Section 4.5 states, in pertinent part (emphasis added in bold.): “The Contractor must contact DEPTCOR to coordinate all aspects of the delivery, including, but not limited to, the delivery timeframes and security measures for delivery. Upon Contract award, the Award Summary for T2854 will list the specific DEPTCOR contacts and e-mail addresses.” |
| 37 | Delivery Schedule, Bid Solicitation Section 4.5 | Question: This section indicates that NJMVC/DEPTCOR will provide a projected schedule to the Contractor not less than thirty (30) business days in advance of the required delivery date. Please confirm that the intent of the projected schedule is to provide the Contractor with the schedule for the entire year, and not just 30 (thirty) business days in advance of each delivery date. Answer: The projected schedule referenced in this section is for the turnaround time and delivery of standard passenger license plates only, which is required to be produced and delivered two (2) times a week. This schedule is continuous in that production and delivery is anticipated two (2) times per week. The main series of motorcycle license plates are ordered once per year. All other license plates will be ordered as needed. Please refer to Bid Solicitation Section 4.5. |
| 38 | Reporting, Bid Solicitation Section 4.6.1 Purpose, and Intent and Background, Bid Solicitation 1.1 Estimated Ordering Quantity, Bid Solicitation 4.17 | Question: Please advise as to the State’s expectations regarding minimum/maximum levels around inventory levels for aluminum alloy coil stock, unprinted reflective sheeting, and printed reflective sheeting at the Contractor’s location. Answer: The following inventory list is kept on hand at DEPTCOR. The Contractor must have at least, if not more, enough inventory on hand to meet the amount that DEPTCOR has: Inventory Type; Part No.; Description; Quantity on Hand; Unit of Measure. Raw Material; 11-0029-4363-4; Aluminum,3105 H-12, .027 in. x 12.00 in. x coil, (PASS); 103,410; Pounds. Raw Material; 11-0029-4367-5; Aluminum,3105 H-12, .027 in. x 7.00 in. x coil, (MC); 0; Pounds. Sheeting Finished Material; 42-0026-7047-1; NJ DLPP PASS VST/ENS DLP/OVERLAY PRINTED; 20; Rolls. Sheeting Finished Material; 42-0026-6991-1; NJ DLPS2P SPECIALTY SHTG/OVERLAY PRINTED; 0; Rolls. Sheeting Finished Material; 42-0026-7046-3; NJ DLPMCP MC VST/ENS DLP/OVERLAY PRINTED; 0; Rolls. Finished Goods; 75-0302-4271-5; DLPNJP GARDEN STATE 2C 6IN X 12IN PAIR; 14,250; Pair. Finished Goods; 75-0302-4272-3; DLPNJC COMMERICIAL 2C 6IN X 12IN PAIR; 0; Pair. Finished Goods; 75-0302-4284-8; DLPNJT TRAILER 2C 6IN X 12IN EA; 0; Each. Finished Goods; 75-0302-4275-6; DLPNJM MOTORCYCLE 2C 4IN X 7IN EA; 0; Each. Finished Goods; 75-0302-4276-4; DLPNJS2 SPECIALTY MULTI C 6IN X 12IN PR; 0; Pair. Finished Goods; 75-0302-4277-2; DLPNJS1 SPECIALTY MULTI C 6IN X 12IN EA; 0; Each. Packaging; 34-8713-7575-3; Polybags For Passenger Plates (OVERSIZED BAGS); 279,600; Each. Packaging; xx-xxxx-xxxx-x; Polybags For Motorcycle Plates; 7,150; Each. Packaging; 34-8713-7580-3; Corrugated Passenger Plate Boxes; 10,100; Each. Packaging; 34-8715-2802-1; Corrugated Specialty Plate Boxes; 4,000; Each. Packaging; 34-8709-6937-4; Corrugated Motorcycle Cartons (REVISED) 6/17/19; 910; Each. Packaging; xx-xxxx-xxxx-x; Tape; 36; Rolls. Packaging; xx-xxxx-xxxx-x; Plate Dividers (Specialty Plates); 16; Box. Packaging; xx-xxxx-xxxx-x; Stretch Wrap (12" x 1,200 LNFT.); 34; Rolls. Packaging; 34-8714-2983-2; Envelopes (Specialty Plates); 67,500; Each. Packaging; xx-xxxx-xxxx-x; Strapping; 12; Rolls. Packaging; xx-xxxx-xxxx-x; Strap Seals; 9; Box. However, the above chart does not guarantee the minimum, average or maximum volume of purchases to be made under this Contract. Bid Solicitation Sections 1.1 and 4.17 state, in pertinent part: “The State makes no representation and provides no guarantee as to the minimum, average or maximum volume of purchases to be made under this Contract.” |
| 39 | Data Management, Security of Data and Materials, Bid Solicitation Section 4.7 | Question: It is understood that the State will not be sending the Contractor any sensitive data information, such as motorist data. The only data being sent to the Contractor would be the number of each plate type to print and the alpha-numeric message to print, which would not generally be considered as sensitive data. Please confirm that this is correct. Answer: Yes, this is correct. |
| 40 | Current Manufacturing Process, Bid Solicitation Section 4.8 | Question: The DEPTCOR embossed process diagram shows a hot water bath being used as part of the application process, presumably for heating the aluminum substrate to enhance the adhesion of reflective sheeting to the aluminum substrate. Please confirm if a hot water bath is still being used. Question: If a hot water bath is still being used, it can introduce impurities between the aluminum and the reflective sheeting which impacts adhesion and warranty. Please confirm that DEPTCOR is responsible for ensuring that the water is clean and free of impurities and that all aluminum is completely dry and clean before the reflective sheeting is applied to it. Answer: Please refer to the answer to question #17 above. The “DEPTCOR Embossed Process” chart has been deleted from Bid Solicitation Section 4.8. Please refer to Part 2 of this Bid Amendment and the revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026”. |
| 41 | Current Manufacturing Process, Bid Solicitation Section 4.8 | Question: If 100% of New Jersey plates are to be digitally printed by the Contractor, including the plate alphanumeric sequence, why is the DEPTCOR embossed process diagram showing embossing machines for embossing personalized and motorcycle plates, or is this an oversight on the process flow diagram? Answer: Please refer to the answer to question #17 above. |
| 42 | Current Manufacturing Process, Bid Solicitation Section 4.8 | Question: If 100% of New Jersey plates are to be digitally printed by the Contractor, why is Step 3 of the DEPTCOR embossed process diagram showing painting, drying racks, drying oven, etc., or is this an oversight on the process flow diagram? Answer: Please refer to the answer to question #17 above. |
| 43 | Current Manufacturing Process, Bid Solicitation Section 4.8 | Question: If 100% of New Jersey plates, including specialty plates, are to be digitally printed by the Contractor, why is Step 2 of the DEPTCOR embossed process diagram showing screen-printing (silk screening) of organizational plates, or is this an oversight on the process flow diagram? Answer: Please refer to the answer to question #17 above. |
| 44 | State-Supplied Price Sheet Instructions, Bid Solicitation Section 3.36 | Question: Please confirm that DEPTCOR is responsible for supplying all packaging materials, labels, etc. used for packaging/palletizing of finished plates, and that the Contractor is not responsible for supplying any materials other than those listed on the pricing table. Answer: The Contractor shall supply DEPTCOR with packaging materials, such as license plate envelopes (a paper barrier for in between all license plate pairs), license plate bags, and labels. The Bidder must include additional costs, including but not limited to costs for packaging materials, in its Quote pricing on the State-Supplied Price Sheet. Bid Solicitation Section 3.36, Step 3, has been updated to include a statement regarding this. Please refer to Part 2 of this Bid Amendment and the revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026”. |
| 45 | ## Reserved, ## Bid Solicitation Section 4.9 ## Price Adjustments for Price Lines 1 through 5, ## Bid Solicitation Section 4.16 | Question: Aluminum price adjustments using the London Metal Exchange (LME) no longer keep pace with aluminum adjustments from U.S. aluminum mills for the specialized aluminum coil required for license plate production. Would the State consider switching to an aluminum adjustment formula where price adjustments are based on the actual amount paid per pound for aluminum, with supplier documented mill quotes/invoices from the aluminum mill as supporting documentation? Answer: For all price adjustment requests, including but not limited to aluminum alloy, printed reflective sheeting, printing/labor costs and other required materials, the Contract may request price adjustments which will be considered by the Division provided that the Contractor provides proper justification to support the price increase and specifically indicates to which price line(s) the requested price increase is applicable. The language in Bid Solicitation Sections 4.9 and 4.16 has been updated. Please refer to Part 2 of this Bid Amendment and the revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026”. |
| 46 | ## Reserved, ## Bid Solicitation Section 4.9 ## Price Adjustments for Price Lines 1 through 5, ## Bid Solicitation Section 4.16 | Question: If the State is not willing to consider supplier documented mill quotes/invoices from the aluminum mill for the quarterly aluminum adjustments, would the State consider adding the Midwest Aluminum Premium to the aluminum adjustment formula? The Midwest Aluminum Premium is the additional cost per pound of aluminum that buyers in the United States of America pay on top of the global aluminum pricing (London Metal Exchange (LME) price). The Midwest Aluminum Premium has been extremely volatile as of late and it is a direct input cost for the price of aluminum. Answer: For all price adjustment requests, including but not limited to aluminum alloy, printed reflective sheeting, printing/labor costs and other required materials, the Contract may request price adjustments which will be considered by the Division provided that the Contractor provides proper justification to support the price increase and specifically indicates to which price line(s) the requested price increase is applicable. The language in Bid Solicitation Sections 4.9 and 4.16 has been updated. Please refer to Part 2 of this Bid Amendment and the revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026”. |
| 47 | ## Reserved, ## Bid Solicitation Section 4.9 ## Price Adjustments for Price Lines 1 through 5, ## Bid Solicitation Section 4.16 | Question: This section indicates the quarterly price adjustment for aluminum alloy coil stock and printed reflective sheeting (license plate pairs), is calculated utilizing the London Metal Exchange (LME). Please advise how the LME is used to calculate quarterly adjustments for printed reflective sheeting. This adjustment does not seem to be related to license plate sheeting. Answer: For all price adjustment requests, including but not limited to aluminum alloy, printed reflective sheeting, printing/labor costs and other required materials, the Contract may request price adjustments which will be considered by the Division provided that the Contractor provides proper justification to support the price increase and specifically indicates to which price line(s) the requested price increase is applicable. The language in Bid Solicitation Sections 4.9 and 4.16 has been updated. Please refer to Part 2 of this Bid Amendment and the revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026”. |
| 48 | Replacement of Unsatisfactory Product, Bid Solicitation Section 4.14 | Question: Please confirm that DEPTCOR is responsible for any damages to the reflective sheeting or aluminum after receipt of product from the Contractor. Answer: Please see answers to Questions #27, 28 and 30 above. |
| 49 | Delivery Schedule, Bid Solicitation Section 4.5 Delivery Time and Costs, Bid Solicitation Section 5.6 | Question: This section indicates that delivery time is forty-five (45) calendar days after receipt of order. Please confirm that the delivery time of forty-five (45) calendar days ARO is for the Contractor to deliver printed sheeting to DEPTCOR. Answer: The delivery timeframe of forty-five (45) calendar days, as listed in Bid Solicitation Sections 4.5 and 5.6, applies to the first delivery, after the approval of the samples by the NJMVC. For all deliveries after the first delivery, the expected delivery timeframe should not exceed fourteen (14) Calendar Days. Bid Solicitation Sections 4.5 and 5.6 have been updated to include a statement regarding this. Please refer to Part 2 of this Bid Amendment and the revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026”. |
| 50 | State-Supplied Price Sheet Instructions, Bid Solicitation Section 3.36 | Question: Please confirm that freight for rush orders, or special orders that are ordered separately from normal orders, will be prepaid by the Contractor and invoiced back to the State at cost. Answer: Please see the answer to question number 27 above. |
| 51 | State Supplied Price Sheet Instructions, Bid Solicitation Section 3.36 Volume Discounts, Bid Solicitation Section 3.36.3 Technical Evaluation Criteria, Bid Solicitation Section 8.9.2 | Question: There are no volumes listed on the price sheet other than one (1) for each line item. How will the State evaluate each line-item price submitted by bidders? Answer: Please refer to Bid Solicitation Section 8.9.2. Bid Solicitation Section 8.9.2 states (emphasis added in bold): “For evaluation purposes, Bidders will be ranked from lowest to highest according to the total Quote price (sum of the Unit Prices for Price Lines 1 through 5) located on the State-Supplied Price Sheet accompanying this Bid Solicitation.” Bidders must bid on all five (5) price lines on the State-Supplied Price Sheet in order for its Quote to be considered responsive. If a Bidder fails to bid on any of the five (5) price lines on the State-Supplied Price Sheet or is determined to be non-responsive for any of the five (5) price lines, the Bidder’s Quote shall be deemed non-responsive. Price Lines 6 and 7 are net lines and will not be used for evaluation and will be added to the awarded Contract.” The first paragraph of Bid Solicitation Section 8.9.2 has been updated to include the statement: “sum of the Unit Prices for Price Lines 1 through 5”. Please refer to Part 2 of this Bid Amendment and the revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026”. The Bid Solicitation and State-Supplied Price Sheet includes a column for volume discounts, but any such volume discounts will not be part of the Quote evaluation. Please refer to Bid Solicitation Sections 3.36, Step 5, and 3.36.3. |
| 52 | State Supplied Price Sheet Instructions, Bid Solicitation Section 3.36 Purpose, and Intent and Background, Bid Solicitation 1.1 Estimated Ordering Quantity, Bid Solicitation 4.17 | Question: Can the State provide the estimated volumes for each of the line items requested, i.e. how many metric tons required per year? All of this information is necessary to determine the volume requirements per year for each line item and provide the State with the best possible pricing. Answer: Please refer to the answer to Question number 18 above. Question: How many pairs of two (2) color plates per year? Answer: NJMVC produces approximately 1,000,000 to 2,000,000, two (2) color license plate pairs per year. Question: How many pairs of multi-color plates per year? Answer: NJMVC produces approximately 50,000 to 120,000 multi-color license plate pairs per year. Question: How many two (2) color single plates per year? Answer: NJMVC produces approximately 110,000 two (2) color single license plates per year. Question: How many multi-color single plates per year? Answer: None. The above estimates do not guarantee the minimum, average or maximum volume of purchases to be made under this Contract. Bid Solicitation Sections 1.1 and 4.17 state, in pertinent part: “The State makes no representation and provides no guarantee as to the minimum, average or maximum volume of purchases to be made under this Contract.” |
| 53 | State Supplied Price Sheet Instructions, Bid Solicitation Section 3.36 Purpose, and Intent and Background, Bid Solicitation 1.1 Estimated Ordering Quantity, Bid Solicitation 4.17 | Question: Can the State please provide a column indicating the number of plates/items to be ordered per year for each line item, and a column for the extension of the number of plates multiplied by the line-item price, as well as a grand total for the line items? Answer: The State will not be making this update, since it is unknown at this time the exact number of license plates that will be ordered by the NJMVC under this Contract. Please also refer to the answer to Question number 18 above. Bidders are reminded of the language in Bid Solicitation Sections 1.1 and 4.17, which state, in pertinent part: “The State makes no representation and provides no guarantee as to the minimum, average or maximum volume of purchases to be made under this Contract.” |
| 54 | ## Reserved, ## Bid Solicitation Section 4.9 ## Price Adjustments for Price Lines 1 through 5, ## Bid Solicitation Section 4.17 | Question: Can the State modify the price sheet per the previous question above, and provide an opportunity to include different pricing in years 2, 3 and 4 of the initial contract term? Answer: No, the State will not modify the State-Supplied Price Sheet to include additional columns or price lines for yearly pricing as part of the initial Contract term, since the Contract provides a Contractor with the ability to adjust its pricing on a quarterly basis, in accordance with Bid Solicitation Section 4.17. |
| 55 | ## Price Adjustments for Price Lines 1 through 5, Bid Solicitation Section 4.17 | Question: Please confirm that documented price adjustments are permitted for the three (3) one (1) year renewal options and confirm how are they handled? Answer: Yes, price adjustments in Bid Solicitation Section 4.16 are applicable during the three (3) one-year extensions referenced in Bid Solicitation Section 5.1. |
| 56 | Aluminum Alloy Coil Stock (Price Line 1), Bid Solicitation Section 4.1 Purpose, and Intent and Background, Bid Solicitation 1.1 Estimated Ordering Quantity, Bid Solicitation 4.17 | Question: How many metric tons of aluminum does the State require per year for the 12-inch plates and 7-inch plates respectively? Answer: Please refer to the answer to Question number 18 above. Bid Solicitation Sections 1.1 and 4.17 state, in pertinent part: “The State makes no representation and provides no guarantee as to the minimum, average or maximum volume of purchases to be made under this Contract.” |
| 57 | Electronic Question and Answer Period, Bid Solicitation Section 2.1 | Question: The bid specifies that the due date for electronic questions is November 19th; however, no date is provided for when responses to those questions will be issued. As the State’s responses may prompt follow-up questions, would the State consider allowing an additional question and answer period following the release of the initial answers to the questions? Answer: Yes, the Question and Answer Period has been re-opened to address any follow up questions resulting from the questions and answers in this Bid Amendment. Please refer to Page 1 of this Bid Amendment for the revised Question and Answer Due date, Part 2 of this Bid Amendment and the revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026”. Please thoroughly review the questions and answers herein. Do not submit any questions that were previously asked and answered through this Bid Amendment. Any such questions will not be addressed. |
| 58 | Aluminum Alloy Coil Stock (Price Line 1), Bid Solicitation Section 4.1 Finished Plate Dimensions, Bid Solicitation Section 4.1.1 | Question: Section 4.1 – Aluminum alloy coil stock width was given as 12.125 inches +/-.030”. Section 4.1.1 – Plate size is listed as 12 inches wide by 6 inches long. Please confirm the exact finished size of the license plate, as well as the exact slit width expected for the reflective sheeting. Answer: The exact size of the finished standard license plate is 12 inches wide x 6 inches long. The exact slit width size is 12 inches. The slit width in Bid Solicitation Section 4.1 has been revised. This has also been addressed in Question and Answer #14 above. Please refer to Part 2 of this Bid Amendment and the revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026”. |
| 59 | Aluminum Alloy Coil Stock Delivery Preparation, Bid Solicitation Section 4.1.2 Purpose, and Intent and Background, Bid Solicitation 1.1 Estimated Ordering Quantity, Bid Solicitation 4.17 | Question: Please provide detail as to the preferred volume per delivery, and frequency, for the aluminum coil stock. Answer: The exact volume per order/delivery, the frequency of orders and the exact order quantity are unknown at this time. Please refer to the answers to question numbers 18 and 38 above. Bidders are reminded that Bid Solicitation Sections 1.1 and 4.17 state, in pertinent part: “The State makes no representation and provides no guarantee as to the minimum, average or maximum volume of purchases to be made under this Contract.” |
| 60 | Graphic Design Work, Bid Solicitation Section 4.2.1 | Question: Since NJMVC is in ownership of all images and license plate graphics, would the State please send high resolution design files for the prospective bidders to use in creating samples? Answer: The State will provide high resolution design files to the awarded Contractor, only, prior to the commencement of the project. Bid Solicitation Section 4.2.1 has been updated to reflect this addition. Please refer to Part 2 of this Bid Amendment and the revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026”. Question: Would the State also provide physical license plate samples to prospective bidders for use in matching the design and colors? Answer: Yes, upon request, NJMVC can provide samples of standard license plates to the Bidders. Bid Solicitation Section 4.2.1 states, in pertinent part: “The NJMVC will be able to provide a sample standard license plate and standard motorcycle plate upon request. Bidders interested in obtaining a sample must do so by contacting: NJMVC P.O. Box 015 225 East State Street Trenton, NJ 08666 Contact Person: Jason Bushnell E-mail address: MVCMOSMGRoffice@mvc.nj.gov Please note that contact must be limited to the request for samples only. Any procurement-related questions will not be answered. Please refer to Bid Solicitation Section 2.1 (Electronic Question and Answer Period) for the procedure involving questions and answers regarding the Bid Solicitation.” |
| 61 | Specifications, Bid Solicitation, Section 4.2.2(D) | Question: Letter D describes a product that is produced using a thermal transfer printer. Must the prospective bidder utilize a thermal transfer printer? Answer: Yes, a thermal transfer printer must be utilized. The thermal transfer printer is the machine that adds color, including the yellow background, the alphanumeric sequence, and any graphics onto the white sheeting roll. Bid Solicitation Section 4.2.2 states, in pertinent part (emphasis added in bold): “The Contractor-provided reflective sheeting, when bonded to the plate and blanked to its finished size, must include: - A protective clear film shall be laminated to the visible surface of the reflective sheeting in-line with the thermal transfer printing process.” (emphasis added in bold) Bid Solicitation Section 9.2 defines the words “must” and “shall” as (emphasis added in bold): “Denotes that which is a mandatory requirement.” |
| 62 | Current Manufacturing Process, Bid Solicitation Section 4.8 Finished Plate Dimensions, Bid Solicitation Section 4.1.1 | Question: Please provide details for current blanking line, i.e. model and serial number for each component. Answer: Please see the answer to Question number 12 above. Bid Solicitation Section 4.1.1 states, in pertinent part: “DEPTCOR utilizes a Federal brand machine- blanking press Model Number 60-OBI, manufactured by the Federal Press Company.” Question: Does the State and/or DEPTCOR own the line in its entirety? Answer: DEPTCOR only owns the blanking machine. |
| 63 | Current Manufacturing Process, Bid Solicitation Section 4.8 | Question: These steps outlined in the diagram appear to reference the process of embossing, numeral coating and curing plates using a solvent oven. In no other section of the Bid Solicitation is this process mentioned or described. Does the State intend to issue any finished license plates as embossed? Question: If so, please describe, in detail, the embossing process and tooling utilized (model/serial numbers etc.). Answer: Please refer to the answer to question #17 above. Please see the answer to question number 41 above. |

## Bid Solicitation # 25DPP01178

T2854 – License Plate Materials, Manufacturing, Printing and Related Services for the New Jersey Motor Vehicle Commission (NJMVC)

Part 2 - Revisions to the Bid Solicitation

Please refer to the Revised Bid Solicitation titled “T2854 Revised Bid Solicitation 3.17.2026” in the Attachments tab in NJSTART.

| # | Bid Solicitation Section Reference | Revision |
| --- | --- | --- |
| 1 | Bid Solicitation Cover Page, Due Date for Electronic Questions | On the cover page of the Bid Solicitation, under “Due Date For Electronic Questions”, the due date for electronic questions has been revised to April 8, 2026. |
| 2 | Bid Solicitation Cover Page, Quote Opening Date | On the cover page of the Bid Solicitation, under “Quote Opening Date”, the bid opening date has been revised to April 24, 2026. |
| 3 | Bid Solicitation Section 1.1 | The below Price Line chart has been modified to delete Price Line 6 and mark same as “Reserved”, and to add Price Lines 8 and 9: Price Line Number; Description. 1; Aluminum Alloy Coil Stock (Metric Ton), as specified in Bid Solicitation Section 4.1.. 2; Printed Reflective Sheeting 2-Color (Plate Pair), as specified in Bid Solicitation Section 4.2.. 3; Printed Reflective Sheeting Multi-Color (Plate Pair), as specified in Bid Solicitation Section 4.2. 4; Printed Reflective Sheeting 2-Color (Single Plate), as specified in Bid Solicitation Section 4.2. 5; Printed Reflective Sheeting 2-Color Motorcycle Plate (Single Plate), as specified in Bid Solicitation Section 4.2. 6; Agency Use Net Line – RESERVED Quarterly Price Adjustment for Aluminum Alloy Coil and Printed Reflective Sheeting Utilizing the London Metal Exchange, as specified in the Bid Solicitation Sections 4.9 and 4.14.. 7; Agency Use Net Line - For Project Management and Technical/Support Services, as specified in Bid Solicitation Sections 4.6 and 4.11.. 8; Agency Use Net Line - For the payment of Expedited Delivery Charges, as specified in Bid Solicitation Section 4.12.. 9; Agency Use Net Line - For the payment of Blanking Line/Machine Repair Services, Maintenance and/or Replacement Parts, as specified in Bid Solicitation Section 4.13.. |
| 4 | Bid Solicitation Section 1.2.1 | The fourth sentence is Bid Solicitation Section 1.2.1 has been revised as follows: “This Bid Solicitation includes language pertaining to the quarterly price adjustments. utilizing the London Metal Exchange. Please refer to Bid Solicitation Section 4.16. |
| 5 | Bid Solicitation Section 3.36 | The below Price Line chart has been modified to delete Price Line 6 and mark same as “Reserved”, and to add Price Lines 8 and 9: Price Line Number; Description. 1; Aluminum Alloy Coil Stock (Metric Ton), as specified in Bid Solicitation Section 4.1.. 2; Printed Reflective Sheeting 2-Color (Plate Pair), as specified in Bid Solicitation Section 4.2.. 3; Printed Reflective Sheeting Multi-Color (Plate Pair), as specified in Bid Solicitation Section 4.2. 4; Printed Reflective Sheeting 2-Color (Single Plate), as specified in Bid Solicitation Section 4.2. 5; Printed Reflective Sheeting 2-Color Motorcycle Plate (Single Plate), as specified in Bid Solicitation Section 4.2. 6; Agency Use Net Line – RESERVED Quarterly Price Adjustment for Aluminum Alloy Coil and Printed Reflective Sheeting Utilizing the London Metal Exchange, as specified in the Bid Solicitation Sections 4.9 and 4.14.. 7; Agency Use Net Line - For Project Management and Technical/Support Services, as specified in Bid Solicitation Sections 4.6 and 4.11.. 8; Agency Use Net Line - For the payment of Expedited Delivery Charges, as specified in Bid Solicitation Section 4.12.. 9; Agency Use Net Line - For the payment of Blanking Line/Machine Repair Services, Maintenance and/or Replacement Parts, as specified in Bid Solicitation Section 4.13.. |
| 6 | Bid Solicitation Section 3.36, Step 3 | Bid Solicitation Section 3.36, Step 3, has been revised to include the following statement (in bold): “Any additional costs, including freight and packaging materials, must be included in the pricing offered on the State-Supplied Price Sheet. The Contractor shall supply DEPTCOR with packaging materials, which includes license plate envelopes (a paper barrier for in between all license plate pairs), license plate bags, and labels.” |
| 7 | Bid Solicitation Section 4.1 | The below chart in Bid Solicitation Section 4.1 has been revised to include temper size “3105 H-21” and to delete the width of “12.125” inches: Aluminum Alloy Coil Stock - Temper:; 3105 H-11, 3105 H-21, 3105 H-111, 3105 H-12, 3105 H-121, 8112 H-11, 8112 H-111, 8112 H-12, or 8112 H-121. Thickness:; 0.027 inches +/-0.002 inches. Width:; 12 inches +/-0.030 inches. Ultimate Strength:; 17,000 to 30,000 PSI. Minimum Yield Strength; 10,000 PSI. Elongation (2" std. Specimen); 10% minimum. Micro-hardness (Brinell); 35 to 50 BHN (40 nominal) or 20 to 60 Rockwell "E" (40 nominal). |
| 8 | Bid Solicitation Section 4.1 | The last sentence of the third paragraph in Bid Solcitation Section 4.1 has been deleted as follows: “Aluminum alloy coil stock temper and tensile properties must conform to the specified chemical composition and tensile strength in accordance with ASTM B-209, latest revision. The aluminum alloy coil stock shall have the properties necessary to provide satisfactory results when individually pierced and embossed to a maximum of 0.090 inches (0.070 inches to 0.075 inches typical) using male and female steel dies (typically hinged) to form license plates without exhibiting stress cracks or other fracture type failures.” |
| 9 | Bid Solicitation Section 4.1 | The following sentence has been added to Bid Solicitation Section 4.1 under “Material Preparation”: “Trivalent chrome conversion coating is acceptable.” |
| 10 | Bid Solicitation Section 4.2.1 | The following sentence (in bold) has been added to first paragraph of Bid Solicitation Section 4.2.1: “The Contractor must design and produce printed reflective sheeting for license plates. All images and graphics for license plates are the property of the NJMVC and shall remain the property of the NJMVC following Contract expiration or Contract termination for cause or convenience. Images, graphics and artwork files will be provided to the awarded Contractor only.” |
| 11 | Bid Solicitation Section 4.3.3.4 | The chart in Bid Solicitation Section 4.3.3.4, also listed below, has been revised to correct the units of measure from degrees to cd/lux/m^2: Color; Entrance Angle. -4 cd/lux/m^2; 40 cd/lux/m^2. White; 50 cd/lux/m^2; 16 cd/lux/m^2. Yellow; 25 cd/lux/m^2; 10 cd/lux/m^2. Orange; 25 cd/lux/m^2; 10 cd/lux/m^2. Lemon Yellow; 25 cd/lux/m^2; 10 cd/lux/m^2. Gold; 25 cd/lux/m^2; 10 cd/lux/m^2. Green; 18 cd/lux/m^2; 7 cd/lux/m^2. Blue; 18 cd/lux/m^2; 3 cd/lux/m^2. Red; 9 cd/lux/m^2; 3 cd/lux/m^2. |
| 12 | Bid Solicitation Section 4.4.1 | The third paragraph of Section 4.4.1 was deleted as follows: The Contractor must develop and submit a recommended best practice draft disaster recovery plan “A” to the State Contract Manager in advance of any proposed project or Contract kick-off meeting, to identify locations, systems, procedures and processes to ensure, in the event of any natural or man-made disaster, or the Contractor’s primary location is rendered unusable for any reason, all Contract deliverables and performance thereof continues without interruption. In the event of such disaster, determination or rendering of the Contractor’s primary business location as “unusable”, the Contractor may be required to implement its State Contract Manager-approved disaster recovery plan to continue uninterrupted operations and satisfy all terms and conditions of any Contract resulting from this Bid Solicitation within twenty-four (24) hours. The Contractor must revise and include any necessary adjustments to its draft disaster recovery plan based on State Contract Manager guidance and input during the Contract kick off meeting. The Contractor may also suggest and/or propose to the State Contract Manager any adjustments and/or improvements it believes necessary to optimize the disaster recovery plan and maximize benefit to NJMVC. The Contractor must submit and request the written State Contract Manager approval of its draft disaster recovery plan “A” within thirty (30) Calendar Days following any initial project or Contract kick off meeting and prior to its disaster recovery plan “A” implementation. The State Contract Manager will not unreasonably withhold review or approval of the Contractor draft disaster recovery plan “A”. NJMVC reserves the right to inspect the Contractor disaster recovery site, in accordance with Bid Solicitation Section 8.7. The Contractor shall be held accountable for data recovery to the point of failure, with no data loss from any scheduled or unscheduled outages. In the event a disaster or assembly malfunction impacts DEPTCOR’s normal operation stopping production of license plates, the Contract shall be prepared to deliver contracted materials to DEPTCOR’s Delaware Disaster Recover site. In the event a disaster or assembly malfunction impacts DEPTCOR’s normal operation stopping production of license plates, the Contractor shall be prepared to deliver fully assembled license plates from their facility directly to NJMVC pursuant to Disaster Recovery Plan B set forth in Bid Solicitation Section 4.4.2. In addition, the Contractor shall be responsible for timely delivery of all materials and ensure shipment is not delayed due to unforeseen circumstances with freight companies throughout the Contract term, and any extension(s) thereof. |
| 13 | Bid Solicitation Section 4.4.2 | The stricken wording was deleted from the first paragraph of Bid Solicitation Section 4.4.2: The Contractor must develop and submit a draft secondary disaster recovery plan “B” to the State Contract Manager in advance of any proposed project or Contract kick-off meeting. Upon State Contract Manager approval, the Contractor’s secondary disaster recovery plan “B” shall be utilized in the event a natural or man-made disaster renders or otherwise determines that DEPTCOR’s Tag Shop (Leesburg, New Jersey location) and DEPTCOR’s Delaware disaster recovery site is are inoperable. |
| 14 | Bid Solicitation Section 4.5 | The following language (in bold) has been added to Bid Solicitation Section 4.5: “The Contractor must deliver aluminum alloy coil stock and printed reflective sheeting directly to the DEPTCOR Tag Shop, located in Leesburg, New Jersey. The Contractor’s first delivery shall be no later than forty-five (45) Calendar Days following after the approval of the samples by the NJMVC. The NJMVC will provide official notification to the Contractor once the samples have been approved. All subsequent orders shall be delivered in advance of and in accordance with NJMVC/DEPTCOR’s production schedule. For all deliveries after the first delivery, the expected delivery timeframe should not exceed fourteen (14) Calendar Days. NJMVC/DEPTCOR will provide a projected schedule to the Contractor not less than thirty (30) Business Days in advance of the required delivery date. The Contractor shall be accountable for all printed reflective sheeting from the place of printing to the point of delivery as well as arrange for the delivery of the finished product from DEPTCOR to the NJMVC’s warehouse. After Contract award, the Contractor shall provide a security plan to the State Contract Manager for “chain of custody” during its production, packaging and distribution of the printed reflective sheeting. A label identifying the production run number, lot number, plate series and manufacture date, shall be affixed to the outside of each shipping package by the Contractor.” |
| 15 | Bid Solicitation Section 4.8 | The chart titled “DEPTCOR EMBOSSED PROCESS” was deleted from Section 4.8. |
| 16 | Bid Solicitation Section 4.9 | Section 4.9 has been deleted in its entity and marked as “Reserved”. “4.9 RESERVED” |
| 17 | Bid Solicitation 4.12 | The following Section and paragraph have been added to the Bid Solicitation: ## “4.12 AGENCY NET USE LINE – PRICE LINE 8 – FOR THE PAYMENT OF EXPEDITED DELIVERY CHARGES An Agency-Use Price Line (Price Line 8) will be added to the awarded Contract for the NJMVC’s payment of expedited delivery charges changes for any “rush”/expedited orders. NJMVC will request a Quote from the Contractor in the event that these services are required and negotiate costs associated with this Agency Net Use Line prior to the issuance of a purchase order.” |
| 18 | Bid Solicitation 4.13 | The following Section and paragraph have been added to the Bid Solicitation: ## “4.13 AGENCY NET USE LINE – PRICE LINE 9 – FOR THE PAYMENT OF BLANKING LINE/MACHINE REPAIR SERVICES, MAINTENANCE AND/OR REPLACEMENT PARTS An Agency-Use Price Line (Price Line 9) will be added to the awarded Contract for the NJMVC’s payment of blanking line/machine repair services, maintenance and/or replacements parts. These services may include but are not limited to blanker sharpening and replacement, blanking die parts replacement, costs associated with the repair services, preventative maintenance and the shipping of any replacement parts. NJMVC will request a Quote from the Contractor and negotiate costs associated with this Agency Net Use Line prior to the issuance of a purchase order.” |
| 19 | Bid Solicitation Section 4.16 | Section 4.16 of the Bid Solicitation has been revised as follows: “PRICE ADJUSTMENTS FOR PRICE LINES 1 THROUGH 5 During the term of the Contract, and any extension(s) thereof, the Contract price for aluminum alloy coil and printed reflective sheeting (license plate pairs) will be adjusted, increased or decreased, on a quarterly basis, utilizing the London Metal Exchange: Home ¦ London Metal Exchange This quarterly price adjustment applies to Price Lines 1, 2, 3, 4 and 5. the Contractor may request price adjustments, increased or decreased, for Price Lines 1 through 5. The Contractor must identify the Price Line(s) in their request, and provide justification with proofs, such as invoices, manufacturer letters, and current industry pricing/exchange rates, which may include but is not limited to the London Metal Exchange and the Midwest Aluminum Premium. Given that the State intends to make one (1) Award for this Contract, the State will monitor commodity pricing and may request pricing decreases if certain materials, including but not limited to aluminum, decrease in price that would be most advantageous to the State. The revised pricing will not be considered valid until it has been reviewed and approved by the Procurement Bureau and published in NJSTART.” |
| 20 | Bid Solicitation Section 4.17 | The following sentence has been revised in Bid Solicitation Section 4.17: “The NJMVC administers approximately 1,100,000 1,500,000 license plate pairs and 80,700 single plates annually from approximately 40 different locations.” |
| 21 | Bid Solicitation Section 5.6 | The following sentences (in bold) have been added to the first paragraph in Bid Solicitation 5.6: “Unless otherwise noted in this Bid Solicitation or on the State-Supplied Price Sheet, all delivery times are forty-five (45) Calendar Days after receipt of order (ARO) for the first delivery and prices for items in Quotes shall be submitted Freight On Board (F.O.B.) Destination. (forty-five (45) Calendar Days ARO/F.O.B) For all deliveries after the first delivery, the expected delivery timeframe should not exceed fourteen (14) Calendar Days. The Contractor shall assume all costs, liability and responsibility for the delivery of merchandise in good condition to the State's Using Agency or designated purchaser. Forty-five (45) Calendar Days ARO/F.O.B. does not cover "spotting" but does include delivery on the receiving platform of the Using Agency at any destination in the State of New Jersey unless otherwise specified. No additional charges will be allowed for any additional transportation costs resulting from partial shipments made at the Contractor’s convenience when a single shipment is ordered. The weights and measures of the State's Using Agency receiving the shipment shall govern. Collect on Delivery (C.O.D.) Terms are not permitted.” The statement “(forty-five (45) Calendar Days ARO/F.O.B.)” has been removed from the paragraph. |
| 22 | Bid Solicitation Section 8.9.2 | The first paragraph of Bid Solicitation Section 8.9.2 has been revised as follows (addition identified in bold): “For evaluation purposes, Bidders will be ranked from lowest to highest according to the total Quote price (sum of the Unit Prices for Price Lines 1 through 5) located on the State-Supplied Price Sheet accompanying this Bid Solicitation.” |
| 23 | Bid Solicitation Section 9.3 | The following definition was added to Bid Solicitation Section 9.3: “Blanking Die – Specialized tools used in sheet metal stamping to cut specific shapes out of a metal sheet.” |
| 24 | State-Supplied Price Sheet | Price Line 6 has been deleted and marked as “Reserved” as follows: “Agency Use Net Line - RESERVED - Quarterly Price Adjustment for Aluminum Alloy Coil and Printed Reflective Sheeting Utilizing the London Metal Exchange, as specified in the Bid Solicitation Sections 4.9 and 4.14. Bidders shall not bid on this price line.” |
| 25 | State-Supplied Price Sheet | Price Line 8 has been added to the State-Supplied Price Sheet for the payment of expedited delivery charges as follows: “Agency Use Net Line - For the payment of Expedited Delivery Charges, as specified in Bid Solicitation Section 4.12.” |
| 26 | State-Supplied Price Sheet | Price Line 9 has been added to the State-Supplied Price Sheet for the payment of blanking line/machine repair services, maintenance and/or replacement parts as follows: “Agency Use Net Line - For the payment of Blanking Line/Machine Repair Services, Maintenance and/or Replacement Parts, as specified in Bid Solicitation Section 4.13.” |